|  | Simulation of hover steps Date: 26 May 2022 Designer: Solidworks Study name: Static 1 Analysis type: Static. Table of Contents Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 5 Mesh information 7 Resultant Forces 8 Beams 8 Study Results 9 Conclusion 11. |
| --- | --- |

# Model Information Model name: hover steps Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Cut-Extrude1; Solid Body; Mass:113.138 kg Volume:0.0419031 m^3 Density:2,700 kg/m^3 Weight:1,108.75 N; C:\Users\JEEVAN\OneDrive\Desktop\preeti proj\hover steps.SLDPRT May 26 11:02:02 2022.

# Study Properties Study name; Static 1. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; FFEPlus. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (C:\Users\JEEVAN\OneDrive\Desktop\preeti proj).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; 6063-T83. Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 2.4e+08 N/m^2. Tensile strength:; 2.55e+08 N/m^2. Elastic modulus:; 6.9e+10 N/m^2. Poisson's ratio:; 0.33. Mass density:; 2,700 kg/m^3. Shear modulus:; 2.58e+10 N/m^2. Thermal expansion coefficient:; 2.34e-05 /Kelvin.; SolidBody 1(Cut-Extrude1)(hover steps). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 2 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); -1.43411; 1,755.48; 1,620.44; 2,389.05. Reaction Moment(N.m); 0; 0; 0; 0.. Fixed-2; Entities:; 2 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); 1.48248; 1,316.97; -1,620.84; 2,088.43. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Distributed Mass-1; Entities:; 3 face(s). Type:; Displacement (Direct transfer). Coordinate System:; Global cartesian coordinates. Translation Values:; ---, ---, --- mm. Rotation Values:; ---, ---, --- deg. Reference coordinates:; 0 0 0 mm. Remote Mass:; 200 kg. Moment of Inertia:; 0,0,0,0,0,0 kg.m^2. Components transferred:; NA.. Gravity-1; Reference:; Top Plane. Values:; 0 0 -9.81. Units:; m/s^2..

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 45.3633 mm. Tolerance; 2.26817 mm. Mesh Quality Plot; High. ## Mesh information - Details Total Nodes; 17982. Total Elements; 8973. Maximum Aspect Ratio; 17.181. % of elements with Aspect Ratio < 3; 28.9. % of elements with Aspect Ratio > 10; 0.825. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:02. Computer name:.

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; 0.0483093; 3,072.45; -0.400116; 3,072.45. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 1.473e+04 N/m^2 Node: 9578; 9.277e+06 N/m^2 Node: 2156. hover steps-Static 1-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0.000e+00 mm Node: 1; 2.182e+00 mm Node: 9771. hover steps-Static 1-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 3.652e-07 Element: 2984; 8.723e-05 Element: 7231. hover steps-Static 1-Strain-Strain1.

# Conclusion